# Verification Plan Document (VPD)

UART Controller Configuration Block

Prepared for Information Technology Institute - Digital IC -Intake 46

Course: Design by Example: Practical ASIC Development

Document Information

| Date | Author |
| --- | --- |
| 15-Feb-2026 | Mohammad Wael |

## 1. Features to Verify

| # | Feature | FRD Requirement | Description / Expected Behavior | Tests | Key Assertions | coverpoints |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Reset behavior | FRD_RTL‑01, FRD_RTL‑02, VFY‑01 FRD‐RTL-07 | Registers to the default value after reset | test_reset_defaults | asser. for checking no x propagation. asser. for checking default values of all regs. | cp. for reset |
| 1.2 | Register map Read/Write operations | FRD‐VFY‐02. | register addresses/fields/access types match specification, including reserved handling. | test_random_write test_random_read test_write_read | asser. for checking no change in reserved bits upon write. | cp. for write addresses cp. for read addresses |
| 1.3 | Protocol rules | FRD‐RTL-04, FRD‐VFY‐03 | RAW hazard handling | test_raw | asser. for checking valid data on RAW | cross cp. for RAW cp. for wr_en |
| 1.4 | ERROR stickiness, W1C behavior | FRD‐RTL-05, FRD‐VFY‐04 | Error is set when and still sticky until software clears it. | test_sticky | asser. for checking clearing sticky bit in reg | cp. for uart_error |
| 1.5 | Shadow behavior for BAUD register | FRD‐REG-07, FRD‐VFY‐04 | BAUD register is updated with BAUD value when update ok is asserted | test_baud_value | asser. for checking update of BAUD reg. | cp. for write addresses cp. for read addresses |
| 1.6 | OOB protection | FRD‐RTL-03, FRD‐RTL-08, FRD‐VFY‐06 | Data integrity maintained | test_oob_read, test_oob_write, | asser. for checking no change on write. asser. for checking read_valid_* =0 | cp. for write addresses cp. for read addresses |
| 1.7 | Busy behavior | FRD‐RTL-08 | Busy bit is only read and not written | test_write_busy test_read_busy | asser. For checking no change at write. | cp. for uart_busy |

## 2. Test Plan

| Test Name | Description |
| --- | --- |
| test_reset_defaults | Apply reset and check known state |
| test_write_read | Write data and read back from same address |
| test_random_write | Random sequence of write. |
| test_random_read | Random sequence of read. |
| test_raw | Write & Read from same address at same cycle |
| test_sticky | Set and clear sticky bit |
| test_baud_value | Change baud rate value |
| test_oob_write | Write to out of bound address |
| test_oob_read | Read from out of bound address |
| test_write_busy | Try to write in busy bit |
| test_read_busy | Read busy when uart_busy =1 |
| test_all_tests | Drive all the previous test sequentially |

## 3. Coverage Goals

| Key | Cover Point | Goal |
| --- | --- | --- |
| C1 | Reset | Check reset is asserted and deasserted |
| C2 | Write address space | All legal address are written |
| C3 | Read address space | All legal address are read |
| C4 (cross) | Read and Write at same address (PORT A) | All addresses hit read & write at same cycle |
| C5 (cross) | Read and Write at same address (PORT B) | All addresses hit read & write at same cycle |
| C6 | uart_error | All values hit. |
| C7 | uart_busy | Check uart_busy is asserted and deasserted |

## 4. Assertions

| Key | Assertion | Purpose |
| --- | --- | --- |
| A1 | Reset clear all Registers | Validate reset behavior |
| A2 | No ‘X’ after reset | Ensure stable output |
| A3 | Software Writes reserved bit | Check no change upon writing reserved bit |
| A4 | Read and Write at same cycle | Ensure software reads new written value. |
| A5 | Assert error then write error register | Ensure sticky bit is 1 until software clears it |
| A6 | Try to clear sticky bit while error is asserted | Ensure error isn’t cleared when software clears it while uart_error isn’t asserted |
| A7 | BAUD value is updated | Check BAUD Value not updated in Active reg until update_ok is asserted |
| A8 | Software writes to OOB address | Ensure no change happens in register file |
| A9 | Software reads from OOB address | Ensure read_valid_* is zero |
| A10 | Software writes busy bit | Check busy bit is not written (RO) |